Methods to obtain air density

1. Hydrostatic Equation:

The air density, r, in kg/m3, can be obtained from

ρ=1g ∆P∆z

where g = 9.81 m/s2 is the acceleration due to gravity;
DP is the measured pressure difference in Pascal units (100 Pa/mb). Dz is the measured height range the sensor was moved in meters.

2. Ideal Gas Law and Measurements from the BME 280 sensor:

The air density, r, in kg/m3, can be obtained from

ρ= PR T

where R = 287.7 J/(kg K) is the gas constant for dry air;
P is the measured pressure in Pascal units (100 Pa/mb).
T  is the measured temperature.